/*

* AF Detection System v5.0

*/

#include <Preferences.h>

#include <BLEDevice.h>

#include <BLEServer.h>

#include <BLEUtils.h>

#include <BLE2902.h>

#include <math.h>

#include "af_detection_esp32.h"

Preferences preferences;

// BLE UUIDs

#define SERVICE_UUID        "4fafc201-1fb5-459e-8fcc-c5c9c331914b"

#define CHAR_UUID_READ      "beb5483e-36e1-4688-b7f5-ea07361b26a8"

#define CHAR_UUID_CONTROL   "beb5483e-36e1-4688-b7f5-ea07361b26a9"

BLEServer*         pServer               = NULL;

BLECharacteristic* pReadCharacteristic   = NULL;

BLECharacteristic* pControlCharacteristic = NULL;

bool deviceConnected    = false;

bool oldDeviceConnected = false;

// Pin configuration

#define ECG_PIN      34

#define LO_PLUS_PIN  26

#define LO_MINUS_PIN 27

// LED Pins

#define GREEN_LED 25

#define RED_LED   33

#define BLUE_LED  32

// Window size — 30 beats matches training window

#define WINDOW_SIZE      30

#define MIN_RR_INTERVAL  300

#define MAX_RR_INTERVAL  2000

// ADAPTIVE THRESHOLD PARAMETERS

#define THRESHOLD_RATIO  0.5f

#define BASELINE_ALPHA   0.005f

#define PEAK_ALPHA       0.05f

#define MIN_AMPLITUDE    100

#define CALIBRATION_MS   5000

// DATA STRUCTURE

struct AFReading {

unsigned long timestamp;

float mean_rr;

float pRR20;

float pRR6_25;

float pRR30;

float pRR50;

float sdsd;

float tpr;

float mean_hr;

uint8_t af_prediction;

uint8_t confidence;

};

// GLOBAL STATE

int   rrIntervals[WINDOW_SIZE];

int   rrIndex    = 0;

int   validBeats = 0;

float currentBPM = 0;

float avgBPM     = 0;

unsigned long lastBlink = 0;

bool  blueState  = false;

// Adaptive threshold state

float adaptiveBaseline  = 1800.0f;

float adaptivePeak      = 2000.0f;

float adaptiveThreshold = 1900.0f;

unsigned long lastPeakTime = 0;

int   lastValue    = 0;

bool  peakDetected = false;

void sendAllReadings();

// BLE CALLBACKS

class MyServerCallbacks : public BLEServerCallbacks {

void onConnect(BLEServer* pServer) {

deviceConnected = true;

Serial.println("Phone connected!");

}

void onDisconnect(BLEServer* pServer) {

deviceConnected = false;

Serial.println("Phone disconnected!");

}

};

class ControlCallbacks : public BLECharacteristicCallbacks {

void onWrite(BLECharacteristic* pCharacteristic) {

String value = pCharacteristic->getValue().c_str();

if (value.length() > 0) {

Serial.print("Received command: "); Serial.println(value);

if (value == "GET_DATA")  sendAllReadings();

else if (value == "CLEAR_DATA") {

preferences.clear();

pReadCharacteristic->setValue("CLEARED");

pReadCharacteristic->notify();

Serial.println("Data cleared via BLE");

} else if (value == "GET_COUNT") {

int count = preferences.getInt("count", 0);

String s  = "COUNT:" + String(count);

pReadCharacteristic->setValue(s.c_str());

pReadCharacteristic->notify();

}

}

}

};

// BLE DATA TRANSFER

void sendAllReadings() {

int total = preferences.getInt("count", 0);

Serial.print("Sending "); Serial.print(total);

Serial.println(" readings...");

for (int i = 0; i < total; i++) {

char key[20];

sprintf(key, "r%d", i);

AFReading reading;

preferences.getBytes(key, &reading, sizeof(AFReading));

String s = String(reading.timestamp)     + "," +

String(reading.mean_hr,   1)  + "," +

String(reading.af_prediction) + "," +

String(reading.confidence);

pReadCharacteristic->setValue(s.c_str());

pReadCharacteristic->notify();

delay(50);

}

pReadCharacteristic->setValue("END");

pReadCharacteristic->notify();

Serial.println("All readings sent!");

}

// LED FUNCTIONS

void allLedsOff() {

digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED,   LOW);

digitalWrite(BLUE_LED,  LOW);

}

void showNormalResult() {

allLedsOff();

digitalWrite(GREEN_LED, HIGH);

Serial.println("LED: GREEN ON (Normal)");

delay(5000);

allLedsOff();

}

void showAFResult() {

allLedsOff();

Serial.println("LED: RED PULSING (Possible AF)");

for (int i = 0; i < 10; i++) {

for (int b = 0;   b < 255; b += 5) { analogWrite(RED_LED, b); delay(10); }

for (int b = 255; b > 0;   b -= 5) { analogWrite(RED_LED, b); delay(10); }

}

allLedsOff();

}

void showProcessing() {

if (millis() - lastBlink > 200) {

blueState = !blueState;

digitalWrite(BLUE_LED, blueState);

lastBlink = millis();

}

}

void showIdle() {

if (millis() - lastBlink > 1000) {

blueState = !blueState;

digitalWrite(BLUE_LED, blueState);

lastBlink = millis();

}

}

void showError() {

allLedsOff();

for (int i = 0; i < 3; i++) {

digitalWrite(GREEN_LED, HIGH);

digitalWrite(RED_LED,   HIGH);

digitalWrite(BLUE_LED,  HIGH);

delay(100);

allLedsOff();

delay(100);

}

}

// FEATURE EXTRACTION

void calculateFeatures(int* rr_intervals, int count, float* features) {

if (count < 5) return;

// Convert to float array for TPR computation

float rr[WINDOW_SIZE];

for (int i = 0; i < count; i++) rr[i] = (float)rr_intervals[i];

// mean_rr

float sum = 0;

for (int i = 0; i < count; i++) sum += rr[i];

float mean_rr = sum / count;

// Successive absolute differences

float diffs[WINDOW_SIZE];

int   dc = count - 1;

for (int i = 0; i < dc; i++)

diffs[i] = fabsf(rr[i+1] - rr[i]);

// pRRx features

int cnt6 = 0, cnt20 = 0, cnt30 = 0, cnt50 = 0;

float sum_sq   = 0;

float diff_sum = 0;

for (int i = 0; i < dc; i++) {

if (diffs[i] > 6.25f)  cnt6++;

if (diffs[i] > 20.0f)  cnt20++;

if (diffs[i] > 30.0f)  cnt30++;

if (diffs[i] > 50.0f)  cnt50++;

diff_sum += diffs[i];

}

float pRR20   = (100.0f * cnt20) / dc;

float pRR6_25 = (100.0f * cnt6)  / dc;

float pRR30   = (100.0f * cnt30) / dc;

float pRR50   = (100.0f * cnt50) / dc;

// SDSD — SD of successive differences

float mean_diff = diff_sum / dc;

float var = 0;

for (int i = 0; i < dc; i++) {

float d = diffs[i] - mean_diff;

var += d * d;

}

float sdsd = sqrtf(var / dc);

// TPR

float tpr = compute_tpr(rr, count);

features[0] = mean_rr;

features[1] = pRR20;

features[2] = pRR6_25;

features[3] = pRR30;

features[4] = pRR50;

features[5] = sdsd;

features[6] = tpr;

}

//   // Debug output

//   Serial.print("  mean_rr : "); Serial.print(features[0], 2); Serial.println(" ms");

//   Serial.print("  pRR20   : "); Serial.print(features[1], 1); Serial.println("%");

//   Serial.print("  pRR6.25 : "); Serial.print(features[2], 1); Serial.println("%");

//   Serial.print("  pRR30   : "); Serial.print(features[3], 1); Serial.println("%");

//   Serial.print("  pRR50   : "); Serial.print(features[4], 1); Serial.println("%");

//   Serial.print("  sdsd    : "); Serial.print(features[5], 2); Serial.println(" ms");

//   Serial.print("  tpr     : "); Serial.print(features[6], 4); Serial.println();

//   Serial.print("  Result  : "); Serial.println(prediction == 1 ? "POSSIBLE AF" : "NORMAL");

//   Serial.print("  Conf    : "); Serial.print(*confidence); Serial.println("%");

//   return prediction;

// }

int predictAF(float* features, int* confidence) {

Eloquent::ML::Port::RandomForest clf;

// Get the raw vote counts from all 3 trees

uint8_t votes[2] = {0};

int prediction = clf.predictWithVotes(features, votes);

// Confidence = winning votes / total trees × 100

*confidence = (int)((float)votes[prediction] / 3.0f * 100.0f);

Serial.print("  Votes    : Normal="); Serial.print(votes[0]);

Serial.print(" | AF=");              Serial.print(votes[1]);

Serial.print(" | Confidence: ");     Serial.print(*confidence);

Serial.println("%");

Serial.print("  Result   : "); Serial.println(prediction == 1 ? "POSSIBLE AF" : "NORMAL");

return prediction;

}

// FLASH STORAGE

void storeReading(float* features, int prediction, int confidence) {

AFReading reading;

reading.timestamp     = millis() /1000;

reading.mean_rr       = features[0];

reading.pRR20         = features[1];

reading.pRR6_25       = features[2];

reading.pRR30         = features[3];

reading.pRR50         = features[4];

reading.sdsd          = features[5];

reading.tpr           = features[6];

reading.mean_hr       = (features[0] > 0) ? 60000.0f / features[0] : 0;

reading.af_prediction = prediction;

reading.confidence    = confidence;

int count = preferences.getInt("count", 0);

char key[20];

sprintf(key, "r%d", count);

preferences.putBytes(key, &reading, sizeof(AFReading));

preferences.putInt("count", count + 1);

Serial.print("Stored reading #"); Serial.println(count + 1);

}

void printStoredReadings() {

int count = preferences.getInt("count", 0);

if (count == 0) { Serial.println("No stored readings."); return; }

Serial.println("\n========================================");

Serial.println("STORED READINGS FROM FLASH");

Serial.println("========================================");

Serial.print("Total: "); Serial.println(count);

for (int i = 0; i < count; i++) {

char key[20];

sprintf(key, "r%d", i);

AFReading r;

preferences.getBytes(key, &r, sizeof(AFReading));

Serial.print("Reading #"); Serial.println(i + 1);

Serial.print("  Timestamp : "); Serial.println(r.timestamp);

Serial.print("  Mean RR   : "); Serial.print(r.mean_rr, 2);  Serial.println(" ms");

Serial.print("  Mean HR   : "); Serial.print(r.mean_hr, 1);  Serial.println(" BPM");

Serial.print("  pRR20     : "); Serial.print(r.pRR20, 1);    Serial.println("%");

Serial.print("  pRR6.25   : "); Serial.print(r.pRR6_25, 1);  Serial.println("%");

Serial.print("  pRR30     : "); Serial.print(r.pRR30, 1);    Serial.println("%");

Serial.print("  pRR50     : "); Serial.print(r.pRR50, 1);    Serial.println("%");

Serial.print("  SDSD      : "); Serial.print(r.sdsd, 2);     Serial.println(" ms");

Serial.print("  TPR       : "); Serial.println(r.tpr, 4);

Serial.print("  Result    : "); Serial.println(r.af_prediction == 1 ? "POSSIBLE AF" : "NORMAL");

Serial.print("  Confidence: "); Serial.print(r.confidence);  Serial.println("%");

}

Serial.println("========================================\n");

}

// ── ADAPTIVE THRESHOLD R-PEAK DETECTION ──────────────────

void detectRPeak(int currentValue, unsigned long currentTime) {

float fval = (float)currentValue;

adaptiveBaseline = adaptiveBaseline + BASELINE_ALPHA * (fval - adaptiveBaseline);

if (fval > adaptiveThreshold)

adaptivePeak = adaptivePeak + PEAK_ALPHA * (fval - adaptivePeak);

float amplitude   = adaptivePeak - adaptiveBaseline;

adaptiveThreshold = adaptiveBaseline + THRESHOLD_RATIO * amplitude;

if (amplitude < MIN_AMPLITUDE) {

peakDetected = false;

lastValue    = currentValue;

return;

}

if ((float)lastValue < adaptiveThreshold &&

fval >= adaptiveThreshold &&

!peakDetected) {

if (lastPeakTime > 0) {

int rrInterval = currentTime - lastPeakTime;

if (rrInterval >= MIN_RR_INTERVAL && rrInterval <= MAX_RR_INTERVAL) {

rrIntervals[rrIndex] = rrInterval;

rrIndex = (rrIndex + 1) % WINDOW_SIZE;

if (validBeats < WINDOW_SIZE) validBeats++;

currentBPM = 60000.0f / rrInterval;

float s = 0;

for (int i = 0; i < validBeats; i++) s += rrIntervals[i];

avgBPM = 60000.0f / (s / validBeats);

Serial.print("Beat ");    Serial.print(validBeats);

Serial.print("/");        Serial.print(WINDOW_SIZE);

Serial.print(" | RR: ");  Serial.print(rrInterval);

Serial.print(" ms | BPM: "); Serial.print(currentBPM, 1);

Serial.print(" | Avg: "); Serial.print(avgBPM, 1);

Serial.print(" | Thr: "); Serial.println((int)adaptiveThreshold);

}

}

lastPeakTime = currentTime;

peakDetected = true;

}

if (fval < adaptiveThreshold - 0.2f * amplitude)

peakDetected = false;

lastValue = currentValue;

}

// ── SETUP ─────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\n================================");

Serial.println("AF Detection System ");

Serial.println("Random Forest Edition");

Serial.println("Trained: MIT-BIH AF Database");

Serial.println("================================\n");

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED,   OUTPUT);

pinMode(BLUE_LED,  OUTPUT);

Serial.println("Testing LEDs ");

digitalWrite(GREEN_LED, HIGH); delay(300); digitalWrite(GREEN_LED, LOW);

digitalWrite(RED_LED,   HIGH); delay(300); digitalWrite(RED_LED,   LOW);

digitalWrite(BLUE_LED,  HIGH); delay(300); digitalWrite(BLUE_LED,  LOW);

Serial.println("LEDs OK\n");

preferences.begin("af-storage", false);

int storedCount = preferences.getInt("count", 0);

Serial.print("Stored readings: "); Serial.println(storedCount);

printStoredReadings();

Serial.println("Initialising BLE ");

BLEDevice::init("AF_Monitor");

pServer = BLEDevice::createServer();

pServer->setCallbacks(new MyServerCallbacks());

BLEService* pService = pServer->createService(SERVICE_UUID);

pReadCharacteristic = pService->createCharacteristic(

CHAR_UUID_READ,

BLECharacteristic::PROPERTY_READ | BLECharacteristic::PROPERTY_NOTIFY

);

pReadCharacteristic->addDescriptor(new BLE2902());

pControlCharacteristic = pService->createCharacteristic(

CHAR_UUID_CONTROL,

BLECharacteristic::PROPERTY_WRITE

);

pControlCharacteristic->setCallbacks(new ControlCallbacks());

pService->start();

BLEAdvertising* pAdvertising = BLEDevice::getAdvertising();

pAdvertising->addServiceUUID(SERVICE_UUID);

pAdvertising->setScanResponse(true);

pAdvertising->setMinPreferred(0x06);

pAdvertising->setMinPreferred(0x12);

BLEDevice::startAdvertising();

Serial.println("BLE Ready! Device: AF_Monitor\n");

pinMode(ECG_PIN,      INPUT);

pinMode(LO_PLUS_PIN,  INPUT);

pinMode(LO_MINUS_PIN, INPUT);

Serial.println("================================");

Serial.println("Calibrating adaptive threshold");

Serial.println("Place fingers on electrodes now");

Serial.println("Hold still for 5 seconds...");

Serial.println("================================");

unsigned long calStart = millis();

unsigned long calEnd   = calStart + CALIBRATION_MS;

bool calBlink = false;

unsigned long lastCalBlink = 0;

while (millis() < calEnd) {

if (millis() - lastCalBlink > 500) {

calBlink = !calBlink;

digitalWrite(BLUE_LED, calBlink);

lastCalBlink = millis();

}

int v = analogRead(ECG_PIN);

adaptiveBaseline = adaptiveBaseline + BASELINE_ALPHA * (v - adaptiveBaseline);

if ((float)v > adaptivePeak) {

adaptivePeak = (float)v;

} else {

adaptivePeak = adaptivePeak * 0.999f;

}

adaptiveThreshold = adaptiveBaseline +

THRESHOLD_RATIO * (adaptivePeak - adaptiveBaseline);

delay(4);

}

digitalWrite(BLUE_LED, LOW);

Serial.print("Baseline  : "); Serial.println((int)adaptiveBaseline);

Serial.print("Peak est  : "); Serial.println((int)adaptivePeak);

Serial.print("Amplitude : "); Serial.println((int)(adaptivePeak - adaptiveBaseline));

Serial.print("Threshold : "); Serial.println((int)adaptiveThreshold);

if ((adaptivePeak - adaptiveBaseline) < MIN_AMPLITUDE) {

Serial.println("WARNING: Low signal amplitude. Check electrodes.");

showError();

} else {

Serial.println("Signal quality: OK");

}

Serial.println("================================");

Serial.println("Measurement starting...");

Serial.println("================================\n");

}

// ── LOOP ──────────────────────────────────────────────────

void loop() {

if (!deviceConnected && oldDeviceConnected) {

delay(500);

pServer->startAdvertising();

Serial.println("Advertising restarted");

oldDeviceConnected = deviceConnected;

}

if (deviceConnected && !oldDeviceConnected) {

oldDeviceConnected = deviceConnected;

}

bool leadsOff = digitalRead(LO_PLUS_PIN) || digitalRead(LO_MINUS_PIN);

if (leadsOff) {

showError();

adaptiveBaseline  = 1800.0f;

adaptivePeak      = 2000.0f;

adaptiveThreshold = 1900.0f;

validBeats = 0;

rrIndex    = 0;

Serial.println("Electrodes disconnected - reset");

delay(1000);

return;

}

int ecgValue        = analogRead(ECG_PIN);

unsigned long nowMs = millis();

if (validBeats < WINDOW_SIZE) {

validBeats > 0 ? showProcessing() : showIdle();

}

detectRPeak(ecgValue, nowMs);

// Analysis triggered after 30 beats

if (validBeats >= WINDOW_SIZE) {

allLedsOff();

Serial.println("\n================================");

Serial.println("30 beats collected — Analysing...");

Serial.println("================================");

// 7 features now

float features[7];

calculateFeatures(rrIntervals, WINDOW_SIZE, features);

Serial.println("\nHRV Features:");

Serial.print("  Mean RR : "); Serial.print(features[0], 2); Serial.println(" ms");

Serial.print("  HR      : "); Serial.print(60000.0f/features[0], 1); Serial.println(" BPM");

Serial.print("  pRR20   : "); Serial.print(features[1], 1); Serial.println("%");

Serial.print("  pRR6.25 : "); Serial.print(features[2], 1); Serial.println("%");

Serial.print("  pRR30   : "); Serial.print(features[3], 1); Serial.println("%");

Serial.print("  pRR50   : "); Serial.print(features[4], 1); Serial.println("%");

Serial.print("  SDSD    : "); Serial.print(features[5], 2); Serial.println(" ms");

Serial.print("  TPR     : "); Serial.print(features[6], 4); Serial.println();

Serial.println();

int confidence;

int prediction = predictAF(features, &confidence);

Serial.println("\n================================");

Serial.println("RESULT:");

Serial.println("================================");

if (prediction == 1) {

Serial.println("RHYTHM: Possible AF Detected");

Serial.println("Consult a healthcare professional.");

showAFResult();

} else {

Serial.println("RHYTHM: Normal Sinus Rhythm");

showNormalResult();

}

Serial.println("================================\n");

storeReading(features, prediction, confidence);

validBeats = 0;

rrIndex    = 0;

delay(5000);

}

delay(4);

}